DECKGRIP® - is an attractive non-slip surfacing material proven throughout New Zealand, Australia and the Pacific under the harshest conditions.  It is soft underfoot, yet rugged for years of wear.  It can be applied to most sealed surfaces, including alloy, fiberglass, painted timber, and painted metal.  For large areas ask for 1000mm x 900mm commercial sheets.  Commercial sheets are deliberately supplied “untrimmed”.  This provides a useful extra “oversize” if required.  See our hand “packs” with pressure sensitive adhesive tape for smaller areas*.

Plain Sheets or with Pressure Sensitive Tape

MARINE – Deckgrip® is lightweight, dampens hull noise, particularly alloy craft.  For all Pleasure Craft, Commercial Work Boats, Trawlers, Lobster/Scallop Boats, Charter/Ferry Operators.

COMMERCIAL – Workshop/Factory floors, Dairy Companies, Meat/Fish processors, Schools, Hospitals, Rest Homes, Motels, Clubs, Swimming Pools and Pedestrian Ramps.  Will withstand light trolley traffic etc…, but is not suited to heavier traffic, i.e. vehicular or trolley jacks in a workshop situation.

HOUSEHOLD – Pathways, steps and stairs, decks, poolside.

IMPORTANT – PREPARATION – Surface to be covered must be sealed, flat, smooth, clean, free of polish, wax finishes, oils etc… and be completely dry.  Remove flaky paint.  Use a cleaning solvent such as Ados Solvent N, or lacquer thinners NOT Mineral Turpentine.  DO NOT apply Deckgrip when the ambient temperature is above 30 degrees Celsius or below 10 degrees Celsius.  It is important that the deck (substrate) temperature does not exceed 35 degrees Celsius.  Dark colours, particularly blue are not recommended for use in higher temperature environments.  Deckgrip® is suitable only for flat or gently curved surfaces.  Because of its excellent resilient shock absorbing properties and inherent memory, it will not handle right angles, tight curves or radii.  Do NOT apply in areas which are constantly underwater i.e. swimming pools or spa pools.

FIXING INSTRUCTIONS – The surface to be covered must be completely clean, dry and free of any film which will spoil the adhesive bond.  If cutting to shape, it may be easier to cut out and use paper patterns first.  Cut final shape with trimming knife or scissors, a small radius at corners avoids scuffing.  IMPORTANT - DO NOT BUTT SHEETS TOGETHER leave a GAP between sheets of Deckgrip.  This creates “channels” to shed water quickly, simplifies matching of the surface patterns, allows for some expansion and contraction when exposed to variation in temperature and is visually attractive.

*PRESSURE SENSITIVE ADHESIVE BACKING - Being a “pressure” sensitive adhesive, it is extremely important to remove the release film progressively, and apply pressure as it is laid, Pressure roll to gain complete bond strength, and to remove air pockets.  This MUST be repeated two or three times with a few hours of adhesion to achieve the necessary bond strength.

It is important to protect the finished edges of Deckgrip® by applying a “bead” of sikaflex 291 sealant or similar.

PLAIN SHEETS – Prepare the Deckgrip® by wiping with a cleaning agent such as Ados Solvent N or lacquer thinners. CAUTION – do not apply liberally, only a light wipe is required.  Excess solvent will damage Deckgrip® and cause it to expand.  For bonding we recommend Ados F38 or Anchor-Weld N 102-L (both are one part rubber based contact adhesives) or, a marine grade 2 part epoxy.  Other adhesives should only be used after checking the suitability of Deckgrip® and the substrate.  For all adhesives, the adhesive manufacturer’s specific instructions relative to the type of substrate being used must be complied with.  It is important to protect the finished edges of Deckgrip® by applying a sealing “bead” of Sikaflex 291 sealant or similar.

MATERIAL CONSTRUCTION – Deckgrip® is a specially developed high density, closed cell, cross linked EVA foam, which has a fused surface, and is not absorbent.  It has excellent shock absorption with a prompt memory (recovery) from any deflection due to impact.  It has outstanding U.V. stability, and resists petrol, diesel and oil

splash.  However it is not to be soaked in fuel and spillage should be washed away as soon as possible.

For further information contact Rubbermark Industries Ltd, Auckland, New Zealand.			www.rubbermark.co.nz

Plain Sheets or with Pressure Sensitive Tape